PROJECT REPORT

Industrial Digital Twin for
2,4-Dichlorophenol (DCP) Batch Reactor

Submitted by: Project Team

Academic Year: 2025–2026

Date: May 2026

Guide: [Faculty Name]

Department: [Department Name]

Institution: [College Name]

# ABSTRACT

This project presents the design, development, and implementation of an Industrial Digital Twin System for a 2,4-Dichlorophenol (DCP) batch reactor used in the chemical manufacturing industry. The system creates a real-time virtual replica of a phenol chlorination reactor, enabling remote monitoring, predictive analytics, and operator training without the risks associated with physical plant operations.

The methodology employs a full-stack web architecture with a Python FastAPI backend handling physics-based simulation, Arrhenius chemical kinetics, PID temperature control, and an ISA-88 compliant recipe state machine. The frontend is built using React 19 with Three.js for interactive 3D reactor visualization and Recharts for live telemetry dashboards. Real-time communication is achieved via WebSocket streaming at 10 Hz, while industrial integration is provided through an OPC-UA server (IEC 62541) for connectivity with Distributed Control Systems.

Key results include a fully functional digital twin with 10-phase batch recipe management, ML-based anomaly detection, soft sensor analytics for real-time purity estimation, electronic batch records with SHA-256 hash chaining for 21 CFR Part 11 compliance, role-based access control with JWT authentication, and multi-format report generation (CSV, PDF, DOCX). The system demonstrates that digital twin technology can significantly improve operational visibility, reduce downtime through predictive maintenance, and provide a safe environment for operator training in hazardous chemical processes.

# CHAPTER 1: INTRODUCTION

## 1.1 Background

The chemical manufacturing industry faces significant challenges in process monitoring, quality control, and safety management. Traditional SCADA systems provide limited insight into complex reaction dynamics. The concept of a Digital Twin — a virtual replica that mirrors a physical system's real-time state — has emerged as a transformative technology in Industry 4.0, enabling continuous monitoring, predictive analytics, and what-if scenario analysis.

2,4-Dichlorophenol (DCP) is a critical industrial chemical (CAS 120-83-2) used as an intermediate in the production of herbicides (2,4-D), antiseptics, and pharmaceuticals. It is produced via the electrophilic aromatic chlorination of phenol using chlorine gas (Cl₂) with a Lewis acid catalyst (FeCl₃). The reaction involves three consecutive chlorination steps, where selectivity control is essential to maximize the desired DCP product while minimizing over-chlorination to 2,4,6-Trichlorophenol (TCP).

## 1.2 Problem Statement

Operating a DCP batch reactor involves: (1) Temperature sensitivity — the reaction requires precise control at 75°C ± 2°C; deviations cause selectivity loss. (2) Safety hazards — chlorine gas is toxic, and exothermic runaway is possible. (3) Quality variability — without continuous monitoring, product purity varies between batches. (4) Training limitations — new operators cannot safely practice on live reactors. (5) Lack of real-time analytics — purity is typically measured only post-batch via laboratory analysis.

## 1.3 Objectives

1. Develop a physics-based reactor simulation with Arrhenius chemical kinetics.

2. Implement an ISA-88 compliant recipe state machine with 10 sequential production phases.

3. Create a real-time 3D visualization of the reactor reflecting physical state.

4. Build a live telemetry dashboard with process variable charting.

5. Implement ML-based anomaly detection for predictive maintenance.

6. Develop a soft sensor for real-time purity estimation without laboratory analysis.

7. Provide OPC-UA industrial protocol integration for DCS connectivity.

8. Implement electronic batch records with SHA-256 hash chaining for regulatory compliance.

9. Build role-based access control with JWT authentication.

10. Generate multi-format batch reports (CSV, PDF, DOCX).

## 1.4 Scope

In Scope: Full-stack web application with simulation, visualization, analytics, authentication, industrial protocol integration, and regulatory-compliant batch records.

Out of Scope: Physical hardware interfacing, cloud deployment, mobile native applications.

# CHAPTER 2: LITERATURE REVIEW

## 2.1 Digital Twin Technology

The concept of Digital Twins was introduced by Dr. Michael Grieves at the University of Michigan in 2002. NASA adopted it for spacecraft lifecycle management. In industry, digital twins have evolved from simple 3D models to comprehensive cyber-physical systems integrating real-time data, physics models, and ML.

## 2.2 ISA-88 Batch Control Standard

ISA-88 (ANSI/ISA-88.01) defines a standard for batch control systems including models for physical equipment, procedural control, and recipe management. The standard defines a hierarchical recipe structure: Procedure → Unit Procedure → Operation → Phase. Our system implements Phase-level control with a 10-state sequential state machine.

## 2.3 OPC Unified Architecture

OPC-UA (IEC 62541) is the industrial communication standard replacing legacy OPC-DA. It provides platform-independent, secure, and reliable data exchange. Our system implements an OPC-UA server to expose reactor process variables as browsable nodes for DCS integration.

## 2.4 21 CFR Part 11 Compliance

The FDA's 21 CFR Part 11 regulation governs electronic records and electronic signatures in pharmaceutical manufacturing. Key requirements include audit trails, access controls, and data integrity verification. Our system implements SHA-256 hash chaining for tamper-evident batch records.

## 2.5 Arrhenius Chemical Kinetics

The Arrhenius equation (k = A × exp(−Ea/RT)) describes the temperature dependence of reaction rates. For phenol chlorination, three consecutive reactions occur with different activation energies, making temperature control critical for selectivity optimization.

# CHAPTER 3: SYSTEM DESIGN & ARCHITECTURE

## 3.1 High-Level Architecture

The system follows a client-server architecture with a Python FastAPI backend and React frontend communicating over REST APIs and WebSockets. The backend runs a physics simulation loop at 10 Hz, broadcasting telemetry to all connected clients. An OPC-UA server provides industrial DCS connectivity.

## 3.2 Technology Stack

| Layer | Technology | Version |
| --- | --- | --- |
| Backend | Python + FastAPI + Uvicorn | 3.10+ |
| Frontend | React + Vite | 19.x / 7.x |
| 3D Engine | Three.js / React Three Fiber | Latest |
| Charting | Recharts | 3.x |
| Database | SQLite (SQLAlchemy ORM) | 3.x |
| Industrial | asyncua (OPC-UA) | 0.9+ |
| Auth | python-jose (JWT) + passlib | Latest |
| Reports | fpdf2, python-docx | Latest |
| Styling | TailwindCSS | 3.x |
| Icons | Lucide React | Latest |

## 3.3 Backend Module Structure

| Module | File | Lines | Purpose |
| --- | --- | --- | --- |
| API Server | main.py | 490 | FastAPI app, REST, WebSocket |
| Simulator | simulator.py | 676 | Physics engine, PID, faults |
| Recipe Engine | recipe_engine.py | 460 | ISA-88 state machine |
| Kinetics | kinetics.py | 124 | Arrhenius rate calculations |
| Soft Sensor | soft_sensor.py | 200 | Real-time purity estimation |
| Auth | auth.py | 266 | JWT + RBAC |
| Data Logger | data_logger.py | 472 | Telemetry, CSV/PDF/DOCX |
| Batch Records | batch_record.py | 459 | EBR + SHA-256 hashing |
| OPC-UA | opcua_server.py | 245 | Industrial protocol |
| Database | database.py | 109 | SQLAlchemy models |
| PID Control | controls.py | 260 | PID controller |
| ML | ml.py | 151 | Anomaly detection |

## 3.4 Frontend Component Structure

| Component | File | Purpose |
| --- | --- | --- |
| App Shell | App.jsx | Main layout, routing, auth |
| Login | Login.jsx | Animated login page |
| Role Views | Roles.jsx | Worker/Owner dashboards |
| 3D Reactor | DCPReactorModel.jsx | Three.js reactor model |
| Recipe Panel | RecipePanel.jsx | ISA-88 step visualization |
| AI Panel | AIStatusPanel.jsx | Anomaly alerts, RUL |
| Charts | Charts.jsx | Temperature trend graphs |
| Controls | ControlPanel.jsx | Sliders, toggles |
| API Service | api.js | WebSocket + REST client |

# CHAPTER 4: IMPLEMENTATION

## 4.1 Reactor Physics Simulation

The core simulator (GodModeReactor class, 676 lines) updates at 10 Hz and models temperature dynamics, pressure, vibration, chemical concentrations, and equipment wear. Temperature control uses a PID controller with Kp=2.0, Ki=0.1, Kd=0.5. The simulation loop calculates: ΔT = (Q_reaction + Q_heater − Q_cooling) × dt

## 4.2 Arrhenius Chemical Kinetics

Three consecutive chlorination reactions are modeled using Arrhenius rate law (k = A × exp(−Ea/RT)):

| Reaction | A (L/mol/min) | Ea (J/mol) | ΔH (J/mol) |
| --- | --- | --- | --- |
| Phenol → 2-CP | 2.4×10⁸ | 62,000 | −125,000 |
| 2-CP → 2,4-DCP | 1.1×10⁷ | 68,000 | −118,000 |
| 2,4-DCP → TCP | 3.2×10⁶ | 74,000 | −110,000 |

## 4.3 ISA-88 Recipe State Machine

10-phase sequential execution with safety interlocks:

| # | Phase | Duration | Target | Action |
| --- | --- | --- | --- | --- |
| 1 | Load Phenol | 30s | 25°C | Load 1.0 kg phenol |
| 2 | Add Solvent | 45s | 25°C | Add chlorobenzene |
| 3 | Heating | 120s | 75°C | Ramp to setpoint |
| 4 | Hold Temp | 60s | 75°C | Stabilize ±2°C |
| 5 | Add Catalyst | 30s | 75°C | N-methylaniline |
| 6 | Chlorinating | 120s | 75°C | Cl₂ gas injection |
| 7 | Cooling | 90s | 30°C | Cool for separation |
| 8 | Separating | 60s | 30°C | Product isolation |

## 4.4 Safety Interlocks

| Interlock | Condition | Action |
| --- | --- | --- |
| Discharge Lock | P > 1.2 bar OR T > 60°C | Block discharge valve |
| Cl₂ Lock | Agitator RPM < 50 | Block chlorine valve |
| Chlorination Lock | T ≠ 75°C ± 2°C | Prevent chlorination |
| Pressure Lock | P > 2.5 bar | Block step transitions |
| Dead-Man Switch | Heart rate > 150 BPM | SCRAM shutdown |

## 4.5 Soft Sensor (Virtual Analyzer)

The soft sensor uses Euler integration of concentration ODEs driven by Arrhenius kinetics. It tracks five chemical species: Phenol, 2-Chlorophenol, 2,4-DCP, TCP, and dissolved Cl₂. Outputs include DCP purity (%), TCP impurity (%), phenol conversion (%), and selectivity.

## 4.6 Authentication & RBAC

| Role | Level | Permissions |
| --- | --- | --- |
| Viewer | 0 | View dashboard, telemetry, AI status |
| Worker | 1 | + Start/Stop recipe, control commands, reports |
| Owner | 2 | + Fault injection, batch records, hash verification |

## 4.7 OPC-UA Industrial Integration

Server endpoint: opc.tcp://0.0.0.0:4840/dcp-twin/. Exposes 15 read-only PV nodes and 3 writable SP nodes. Updates at 1 Hz. Compatible with Siemens PCS7, Honeywell Experion, ABB 800xA.

## 4.8 Electronic Batch Records

ISA-88 / 21 CFR Part 11 compliant records with per-phase audit, material balance tracking, quality summary, SHA-256 hash chaining for tamper evidence, JSON persistence, and Markdown export.

## 4.9 3D Reactor Visualization

Built with React Three Fiber: rotation proportional to agitator RPM, color mapping from blue (cold) to red (hot), vibration proportional to bearing wear, interactive orbit camera controls.

# CHAPTER 5: TESTING & RESULTS

## 5.1 Functional Testing

| Feature | Test | Result |
| --- | --- | --- |
| Login | Valid/invalid credentials | ✅ JWT issued / 401 rejected |
| RBAC | Viewer accessing Owner endpoint | ✅ 403 Forbidden |
| Recipe | Full 10-phase execution | ✅ Completes sequentially |
| Interlocks | Cl₂ valve at low RPM | ✅ Blocked with reason |
| SCRAM | Heart rate > 150 BPM | ✅ Emergency shutdown |
| Soft Sensor | Purity during chlorination | ✅ Tracks DCP concentration |
| Reports | CSV/PDF/DOCX generation | ✅ Files generated correctly |
| OPC-UA | Node browsing | ✅ All 18 nodes visible |
| Hash Chain | Tamper detection | ✅ Modified records detected |
| WebSocket | 10 Hz telemetry | ✅ < 50ms latency |

## 5.2 Performance Results

| Metric | Target | Achieved |
| --- | --- | --- |
| Simulation rate | 10 Hz | 10 Hz stable |
| WebSocket latency | < 50ms | ~20ms avg |
| API response time | < 500ms | ~50ms avg |
| 3D render FPS | ≥ 30 FPS | 60 FPS |
| Concurrent connections | 10+ | Tested with 10 |

# CHAPTER 6: API REFERENCE

The system exposes 30+ REST endpoints and 1 WebSocket endpoint:

| Method | Endpoint | Role | Purpose |
| --- | --- | --- | --- |
| POST | /api/auth/login | None | Authenticate user |
| GET | /api/status | Viewer+ | Reactor status |
| GET | /api/recipe/status | Viewer+ | Recipe state |
| POST | /api/recipe/start | Worker+ | Start batch |
| POST | /api/recipe/stop | Worker+ | Abort batch |
| POST | /api/control | Worker+ | Send command |
| GET | /api/soft-sensor | Viewer+ | Purity data |
| GET | /api/batch/report/pdf | Worker+ | PDF report |
| GET | /api/batch/report/docx | Worker+ | DOCX report |
| GET | /api/opcua/status | Viewer+ | OPC-UA info |
| GET | /api/v1/batch-records | Owner | List EBRs |
| POST | /api/v1/batch-records/{id}/verify | Owner | Verify hash |
| WS | /ws | Viewer+ | Live telemetry (10 Hz) |

# CHAPTER 7: CONCLUSION & FUTURE WORK

## 7.1 Conclusion

This project successfully demonstrates the feasibility and value of digital twin technology for chemical batch reactors. The system integrates physics-based simulation, real-time visualization, AI-driven anomaly detection, and regulatory-compliant batch records into a cohesive web application. Key achievements include:

1. A realistic reactor simulation with Arrhenius kinetics updating at 10 Hz.

2. ISA-88 compliant recipe management with safety interlocks.

3. Real-time 3D visualization reflecting physical reactor state.

4. Soft sensor analytics eliminating the need for periodic lab analysis.

5. OPC-UA integration enabling connectivity with industrial DCS platforms.

6. Tamper-evident electronic batch records meeting 21 CFR Part 11 requirements.

7. Role-based access control ensuring operational security.

## 7.2 Future Enhancements

1. Persistent User Database — Migrate to database-backed user management.

2. HTTPS/TLS — Add SSL for encrypted production communication.

3. Time-Series Database — InfluxDB/TimescaleDB for historical trend analysis.

4. Advanced ML — LSTM models for predictive maintenance.

5. Mobile Responsive UI — Optimize dashboard for tablets and phones.

6. Multi-Reactor Support — Monitor and control multiple reactor instances.

7. Email/SMS Alerts — Push notifications for critical anomalies.

8. Docker + Cloud Deployment — Containerize and deploy on AWS/Azure/GCP.

9. PLC Integration — Direct Modbus TCP or OPC-UA client mode.

# REFERENCES

[1] IEEE 830-1998 — Recommended Practice for Software Requirements Specifications.

[2] ISA-88.01 — Batch Control: Models and Terminology.

[3] 21 CFR Part 11 — FDA Electronic Records and Signatures.

[4] IEC 62541 — OPC Unified Architecture Specification.

[5] FastAPI Documentation — https://fastapi.tiangolo.com

[6] React Three Fiber Documentation — https://docs.pmnd.rs/react-three-fiber

[7] Recharts Library — https://recharts.org

[8] Scikit-Learn (Isolation Forest) — https://scikit-learn.org

[9] Grieves, M. (2014). Digital Twins: Excellence through Intelligent Digital Twins.

[10] Python asyncua — https://github.com/FreeOpcUa/opcua-asyncio